FIRE HISTORY

LAYER DESCRIPTION

CUSTODIAN

Technology Solutions Unit

Forest, Fire Regions

Department of Environment, Land Water & Planning

JURISDICTION

State of Victoria

CONTACT INFORMATION

Custodian

Naomi Withers

Technology Solutions Unit

Forest, Fire Regions

Department of Environment, Land Water & Planning

L13, 8 Nicholson St, East Melbourne, 3002

Australia

General information

Naomi Withers phone +61 3 9637 9897

Email: naomi,withers@delwp.vic.gov.au

Supply Enquiries

Fire History data can be ordered direct from Data Vic.

www.data.vic.gov.au

Search for: Fire History Records of Fires primarily on Public Land

GEOGRAPHIC EXTENT

Fire History extends across the whole of Victoria and some fires extend into New South Wales and South Australia.

SPATIAL REFERENCE SYSTEM

The datum used in construction of maintenance of fire history data is the Geocentric Datum of Australia (GDA94). Data is held in geographical (latitude/longitude).

PRODUCTION OR ACQUISITION METHODS

Fire History was originally gathered from a systematic search of fire history maps held in regional offices and the transfer of fire history from available aerial photography. Common acquisition methods used today include remotely sensed satellite/aerial photograph interpretation, GPS fire perimeter and on map digitisation.

SPATIAL ACCURACY

Precision: 10m to 300m - 1:25,000

50m to 300m - 1:100,000

Fire records are aligned to 1:25,000 & 1:100,000 base features. Map errors of 0.5mm to 3mm translate to a positional accuracy of between 10m and 300m, depending on the scale.

ATTRIBUTE ACCURACY

Based on attribute checking procedures, an error of 1 in 20 is reasonable for the fire year, i.e. 95% attribute accuracy.

MAINTENANCE AND UPDATE FREQUENCY

Fire History layers are updated annually on 1 July with the previous fire season’s fire history. Additional versions are created after planned burning season to supplement the Fire History dataset.

GUIDELINES FOR UPDATING FIRE HISTORY

The Code of Practice for Fire Management on public land states that the Department will ensure that adequate records of fire/burns that occur will be collected and stored for future reference. For this to occur the following guidelines have been developed to ensure that adequate bushfire/burn spatial data is recorded.

REGIONAL GIS DATA COLLATION AND QA PROCESS

Fire history Collation Process annually:

- Regional GIS staff to collate fire history for their respective Districts.

  - Schema is located at

  - https://spatial.help.ffm.vic.gov.au/fire-history-help/

  - Fire history & Severity Template for 2021-2022 (file geodatabase) downloadable

  - Data should be audited against Fireweb fire reports and Burns & Works burns completed report (will be FMS for 19/20 season and beyond).

  - Data provided to statewide custodian of Fire History data (regional GIS) for compilation into Statewide data

  - Fires should be audited for quality and completeness by the Fire management officer or delegate.

- Custodian compiles into Statewide dataset

- Fire scars remain the extent of the severity data – to ensure consistency of fire areas. The extent is not recalculated to the burn severity dataset.

- Planned burn treated area can be the burnt area and the unburnt area – severity classification is the only way to discriminate between treatments.

- Following validations run:

  - Check for consistency of attributing

  - Check all attributes align with domain / subtype tables

  - Check and rectify true curves using ArcGIS "Densify" toolset and, by angle, 5°

  - Validate fire and burn names against Fireweb using DSE_ID as primary key against EVENTID

  - Run ArcGIS "Repair Geometry" to fix any null or invalid geometry

  - Run Python script FIRE_HISTORY_TRANSLATE_SEVERITY.py to map fire severity from L1 - L5 fields across to homogenised FIRE_SEVERITY field

- Generate derived products

  - FIRE_HISTORY via definition query FIRE_SEVERITY <> 'UNBURNT'

  - FIRE_HISTORY_LASTBURNT via Python script FIRE_HISTORY_BUILD_LASTBURNT.py

  - FIRE_HISTORY_SCAR via Python script FIRE_HISTORY_CREATE_FIRE_AREA.py

  - FIRE_HISTORY_SCAR_LASTBURNT via Python script FIRE_HISTORY_BUILD_FIRESCAR_LASTBURNT.py

- Both both _LASTBURNT datasets, run LASTBURNT_ELIMINATE_SMALL_POLYGONS.py to eliminate small polygons < 0.25ha

- Run Repair Geometry on all datasets

- Validate event id with fires and burns to check any start dates which were unknown.

- Checked format of the start date.

### FIRE_HISTORY_ORIGIN

- Get list of fires (from FireWeb>Reports>Fires - All Fires 20xx-20xx Season Report), open in ArcMap and create XY events.

- Burns list for that season can be obtained before the 30th June (before the Burn Record Year change over) from the FMS_BURN_ORIGIN using the following query BURN_STATUS in ( 'COMPLETE', 'PART COMPLETE', 'SAFE', 'UNDER CONTROL - 2','UNDER CONTROL - 1','IGNITION').

- Append both sets of data to FIRE_HISTORY_ORIGIN and load into all environments of eMap as well as updating VSDLLD.

UPDATES AFTER OFFICIAL ANNUAL UPDATE

NOTE: There is no official documentation of frequency of update to fire history layers, however due to timelines for Fire Severity with burn seasons, additional updates occur:

- Fire Severity update may occur before Fire season start i.e. end of September and if necessary March.

- This update will need to capture any fires or burns ignited close to 30 June time line.

Upload the above data-sets into eMap Databases

From 2019 and beyond the following Feature Classes will be uploaded to respective SDE  DBs

- FIRE_HISTORY

- FIRE_HISTORY_geog - the records in this feature class is the same as FIRE_HISTORY except its spatial type stored as GEOGRAPHY spatial type. This spatial type is required in consequence mapping script to accurate area statements - more details can be found in - https://desktop.arcgis.com/en/arcmap/10.3/manage-data/gdbs-in-sql-server/sqlserver-spatial-types-and-arcgis.htm

- FIRE_HISTORY_LASTBURNT

- FIRE_HISTORY_SCAR

- FIRE_HISTORY_SCAR_LASTBURNT

- FIRE_HISTORY_TREATED_AREA

#### Run Rapidfire History

#### Yearly update. Base Rapidfire History dataset to be finalised by the first week of August. Liase with Bushfire Risk landscape team for testing.

The purpose of this script is to create a stable featureclass that does not change for that season. It should be created immediately after the end of financial year major update of the fire history dataset and tested by the Bushfire Risk landscape team. Adhoc updates of the fire history dataset are not applied to this dataset.

This script requires two file geodatabases to be available in the same folder as the .py.

- Rapidfire\History.gdb (this can be empty)

- Rapidfire\Rapidfire.gdb (this needs to contain NonTreatableEVCs_S which is a copy of Rapidfire_History\NonTreatableEVCs.shp that has been run through the multipart to singlepart tool)

Outputs a Rapidfire_History_YYYYMMDD.zip file that is to be sent to the BRL team for testing.

#### Other datasets of the same YYYYMMDD in the Rapidfire\History.gdb should also be provided to the BRL so they can verify the correct features have been included in the zipped folder

Currently a treated area process which has been scripted for BRAU – there are some discrepancies between the

ARCHIVING

Archive the following layer from eMap  prdsqlemap04

MAPPING.OBS_AREA

Each year fire history is archived and the new fire history dataset is published in the corporate library.

FUTURE PLANS

Fire history has been further enhanced and developed through the implementation of the following projects:

 Development of a Fuel Management System (FMS) will streamline the process of creating planned and treated areas. The new system will include defined workflows which integrate FMS and eMap. FFMVIC and CFA planned burns will be able to be mapped via eMap with treated area being able to be captured. Severity data will need to be added at a later date through the Fire history collation process.

 eMap Bushfire allows CFA and FFMVic bushfires to be mapped from linescan imagery, and AIG and field data collection tools. Bushfire severity is being captured from Landsat, Sentinel, Linescan and serial photography and is processed in the new GEE tool or Arcgis to be collated post fire season.

 Include the new Burn severity SOP recommendations and align the geodatabase to these streamlined severity classes. These recommendations are waiting for sign off from FFOD.

 Non burn fuel treatments are now being captured, there needs to be work on a new dataset around treatment – which isn’t related to fire history.

 Additional fields need to be included in emap Fuel management around Treated area (NBFT) and (unburnt) to help clarify the status.

| LAYER TABLE SUMMARY Table Name |  |  |  |  | Description |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| FIRE_HISTORY_MASTER |  |  |  |  | This is intended to be the 'point of truth' for all fire history products. This dataset contains geometry for ALL fire history (including unburnt polygons for planned burning activities), for ALL severities, for ALL seasons. This is the source dataset that is edited and modified to create the following products. |  |  |  |
| FIRE_HISTORY |  |  |  |  | This layer represents the spatial extent of fires recorded since 1903 primarily on public land. This layer includes bushfire and planned burn information. CFA fires included since 2009 fire season. |  |  |  |
| FIRE_HISTORY_ORIGIN |  |  |  |  | This layer represents the first reported location of bushfire events. This information is sourced from the DELWP FireWeb and CFA IMS systems. This layer does not represent the exact fire origin. |  |  |  |
| FIRE_HISTORY_LASTBURNT |  |  |  |  | This layer is derived from FIRE_HISTORY and represents the spatial extent of the last fires/burns recorded, primarily on public land. This layer is optimised for map presentation. This layer includes severity and scars. It now includes fires on private land. |  |  |  |
| FIRE_HISTORY_SCAR |  |  |  |  | This layer represents the spatial extent of burnt areas from bushfire or planned burns activity. This layer excludes unburnt areas and is derived from the FIRE_HISTORY Layer. It shows the extent impacted by a fire or planned burning activity, for this purpose all severity polygons are dissolved into a single polygon for each incident. |  |  |  |
| FIRE_HISTORY_SCAR_LASTBURNT |  |  |  |  | This layer represents the spatial extent burnt areas from bushfire or planned burns activity. Its derived from FIRE_HISTORY_SCAR Layer, unburnt areas and doesn’t include severity data. |  |  |  |
| FIRE_HISTORY_TREATED_AREA |  |  |  |  | This layer represents the spatial extent burnt areas from bushfire or planned burns activity, it also includes areas treated which include unburnt areas. |  |  |  |
| Layer Table Structure Table: Fire History (polygon) |  |  |  |  |  |
| Column |  | Data Type |  | Field Size |  |  | NULL | Description |  |
| FIRETYPE |  | VARCHAR |  | 8 |  |  | Y | The characteristics of the fire, if it was unplanned (a bushfire), a planned burn, unknown or ‘none’ i.e. treatments other than fire. |  |
| SEASON |  | NUMBER |  | (4,0) |  |  | Y | Fire Season (Eg. 2012 = July 1 2011 – June 30 2012) |  |
| FIRE_NO |  | VARCHAR |  | 50 |  |  | Y | Fire / planned burn number |  |
| NAME |  | VARCHAR |  | 255 |  |  | Y | Name of fire / planned burn |  |
| START_DATE |  | DATE |  | 7 |  |  | Y | Fire or planned burn ignition date |  |
| START_DATE_INT |  | LONG INT |  |  |  |  |  | Fire or planned burn ignition date as integer data type. Format: YYYYMMDD |  |
| TREATMENT_TYPE |  | VARCHAR |  | 50 |  |  | Y | Activity that occurred within the polygon. |  |
| FIRE_SEVERITY |  | VARCHAR |  | 25 |  |  | Y | The visual impact of the fire on the vegetation, in terms of the vegetation looking black (burnt), brown (scorched) or green (unaffected). |  |
| FIRE_COVER |  | VARCHAR |  | 50 |  |  | Y | Estimated proportion of area that was burnt within the polygon. |  |
| FIREKEY |  | VARCHAR |  | 50 |  |  | Y | Unique identifier for bushfires and planned burns. Eg. W20120312 [Fire Type: W – Bushfire, B – Burn, U - Unknown] [season][district_id][fire_number] |  |
| CREATE_DATE |  | DATE |  | 7 |  |  | Y | Date/time stamped against each feature when it is entered into the database. |  |
| UPDATE_DATE |  | DATE |  | 7 |  |  | Y | The date when the feature was updated. |  |
| AREA_HA |  | DOUBLE |  | (38,8) |  |  | Y | Area of the polygon in hectares (derived from polygon rather than estimated) |  |
| METHOD |  | VARCHAR |  | 50 |  |  | Y | The method used to determine the polygon boundaries |  |
| METHOD_COMMENTS |  | VARCHAR |  | 255 |  |  | Y | Further details about the method used to determine polygon boundaries e.g. which remote sensing platform. |  |
| ACCURACY |  | VARCHAR |  | 50 |  |  | Y | Positional accuracy of input data |  |
| DSE_ID |  | NUMBER |  | 10 |  |  | Y | DSE FireWeb unique fire ID. |  |
| CFA_ID |  | NUMBER |  | 10 |  |  | Y | CFA FIRS System unique fire ID |  |
| DISTRICT_ID |  | VARCHAR |  | 2 |  |  | Y | DSE Fire District identifier |  |

TABLE: FIRE_HISTORY_ORIGIN (point)

| Column | Data Type | Field Size | NULL | Description |
| --- | --- | --- | --- | --- |
| SEASON | NUMBER | (4,0) | Y | Fire/ planned burn season |
| FIRE_NO | VARCHAR | 25 | Y | Fire / planned burn number |
| NAME | VARCHAR | 255 | Y | Name of the fire / planned burn |
| START_DATE | DATE | 7 | Y | Ignition date for fire / planned burn |
| CAUSE | VARCHAR | 100 | Y | Fire cause |
| DSE_ID | NUMBER | 10 | Y | DSE FireWeb unique fire ID |
| CFA_ID | NUMBER | 10 | Y | CFA FIRS System unique fire ID |
| AREA_HA | DOUBLE | (38,8) | Y | Area as reported in DSE/CFA IMS systems |

TABLE: FIRE_HISTORY_LASTBURNT (polygon)

| Column | Data Type | Field Size | NULL | Description |
| --- | --- | --- | --- | --- |
| FIRETYPE | VARCHAR | 8 | Y | Fire type descriptor |
| SEASON | NUMBER | (4,0) | Y | Fire Season |
| FIRE_NO | VARCHAR | 25 | Y | Fire/burn number |
| NAME | VARCHAR | 255 | Y | Name of the fire/burn |
| START_DATE | DATE | 7 | Y | Fire started data |
| PBURN_TYPE | VARCHAR | 50 | Y | Planned burn type |
| DSE_ID | NUMBER | 10 | Y | DSE FireWeb unique fire/burn ID |
| CFA_ID | NUMBER | 10 | Y | CFA FIRS System unique fire ID |
| FIREKEY | VARCHAR | 10 | Y | Redundant fire key from previous Fires system |
| AREA_HA | DOUBLE | (38,8) | Y | Area in hectares |
| CREATE_DATE | DATE | 7 | Y | Date/time stamped against each feature when it is entered into the database. |

LOOK UP TABLE CODE LISTS

FIRETYPE

| FIRETYPE | Description |
| --- | --- |
| BUSHFIRE | Bushfire / Wildfire – a fire that was not planned and was ignited by causes such as lightning, accident or arson |
| BURN | Planned Burn – an approved fire that has been planned and ignited for a management purpose such as reducing levels of fuel |
| UNKNOWN | Unknown Fire Type |
| OTHER | Other treatment type applied – e.g. slashing |

SEASON

| SEASON | Description |
| --- | --- |
| 1903 - Current | Fire season ¦ Eg. 2010 Fire Season is July 1 2009 to June 30 2010 |

TREATMENT_TYPE

| TREATMENT_TYPE | Description |
| --- | --- |
| Fuel Reduction | Fuel Reduction Burn – a burn designed to reduce levels of fuel. Typically fuel reduction burns are classified as ‘low intensity’ |
| Ecological | Ecological Burn – a burn designed to promote ecological values. |
| Other | Other burn type |
| Slash - Ash | Slash - Ash (Excludes Woollybutt) – Slash burns are fires that are ignited for the purposes of removing large quantities of fuel (the ‘slash’) after logging/harvesting operations and for preparing the seed-bed to improve forest regeneration. The ‘slash’ is distributed across the site rather than in heaps or windrows. Typically slash burns are classified as ‘high-intensity’ so they generate as much heat as possible in a short period of time. |
| Slash - Mixed | Slash - Mixed Species – ash, mixed species, softwood and woollybutt are types of forest based on the main species of tree present. Ash, mixed species and woollybutt are eucalypts. |
| Slash - Softwood | Slash - Softwood |
| Slash - Woollybutt | Slash - Woollybutt |
| Slash - Unknown | Slash - Unknown |
| Windrow - Ash | Windrow - Ash (Exclude Woollybutt) |
| Windrow - Mixed | Windrow - Mixed Species |
| Windrow - Softwood | Windrow - Softwood |
| Windrow - Woollybutt | Windrow - Woollybutt |
| Not determined | Not determined |
| Candling | Burns only the bark from the tree trunks, with little or no fire spreading across the ground surface. |
| Slashing | The vegetation is slashed not burnt – no fire is involved |

| FIRE_SEVERITY |  |
| --- | --- |

| FIRE_SEVERITY L1 | Description |
| --- | --- |
| 1_UNBURNT | 1 UNBURNT |
| 1_BURNT | 1 BURNT |
| FIRE_SEVERITY L2 | Description |
| 2_UNBURNT | 2 UNBURNT |
| 2_BURNT_UC | 2 BURNT UNCLASSIFIED |
| 2_BURNT_NF | 2 BURNT NON-FOREST |
| 2_LOW | 2 LOW |
| 2_MED | 2 MEDIUM |
| 2_HIGH | 2 HIGH |
| FIRE_SEVERITY L3 | Description |
| 3_UNBURNT | 3 UNBURNT |
| 3_BURNT_UC | 3 BURNT UNCLASSIFIED |
| 3_BURNT_NF | 3 BURNT NON-FOREST |
| 3_LOW_CS | 3 LOW CANOPY SCORCH |
| 3_MED_CS | 3 MEDIUM CANOPY SCORCH |
| BURNT_2P | 3 MEDIUM CANOPY SCORCH |
| 3_HIGH_CS | 3 HIGH CANOPY SCORCH |
| BURNT_2F | 3 HIGH CANOPY SCORCH |
| 3_CANOPY_BURNT | 3 CANOPY BURNT |
| FIRE_SEVERITY L4 | Description |
| 4_UNBURNT | 4 UNBURNT |
| 4_BURNT_UC | 4 BURNT UNCLASSIFIED |
| 4_BURNT_NF | 4 BURNT NON-FOREST |
| 4_LOW_CS_MIN | 4 LOW CANOPY SCORCH, MIN. UNDERSTOREY BURN |
| 4_LOW_CS_MAJ | 4 LOW CANOPY SCORCH, MAJ. UNDERSTOREY BURN |
| 4_MED_CS_MIN | 4 MED. CANOPY SCORCH, MIN. UNDERSTOREY BURN |
| 4_MED_CS_MAJ | 4 MED. CANOPY SCORCH, MAJ. UNDERSTOREY BURN |
| 4_HIGH_CS |  |
| 4_CANOPY_BURNT | 4 CANOPY BURNT |
| FIRE_SEVERITY L5 | Description |
| 5_UNBURNT | 5 UNBURNT |
| 5_BURNT_UC | 5 BURNT UNCLASSIFIED |
| 5_BURNT_NF | 5 BURNT NON-FOREST |
| 5_GREEN | 5 GREEN |
| 5_BROWN | 5 BROWN |
| 5_BLACK | 5 HIGH |

FIRE_COVER

| FIRE_COVER | Description |
| --- | --- |
| 0-9 | 0-9% estimated burn coverage |
| 10-29 | 10-29% estimated burn coverage |
| 30-49 | 30-49% estimated burn coverage |
| 50-69 | 50-69% estimated burn coverage |
| 70-89 | 70-89% estimated burn coverage |
| 90-100 | 90-100% estimated burn coverage |
| Unknown | Burn coverage not estimated. |

DISTRICT NUMBERS

| Fire District | District No |
| --- | --- |
| BAIRNSDALE | 01 |
| BAW BAW | 02 |
| EAST METROPOLITAN | 03 |
| FAR SOUTH WEST | 04 |
| GOULBURN | 05 |
| HEYFIELD | 06 |
| MALLEE | 07 |
| MIDLANDS | 08 |
| MURRAY GOLDFIELDS | 09 |
| MURRINDINDI | 10 |
| ORBOST | 11 |
| OTWAY | 12 |
| OVENS | 13 |
| SOUTH GIPPSLAND | 14 |
| SWIFTS CREEK | 15 |
| UPPER MURRAY | 16 |
| WIMMERA | 17 |
| YARRA | 18 |

ACCURACY

| Accuracy | Description |
| --- | --- |
| High | 25m or less |
| Medium | 26m to 100m |
| Low | Greater than 100m |
| Unknown | unknown |

METHOD

| Method | Description |
| --- | --- |
| Firescan | Firescan interpretation |
| Air observation | Air observation |
| Aerial photograph interpretation | Aerial Photograph interpretation |
| Satellite image interpretation | Satellite image interpretation |
| Fire Operations Plan | Fire Operations Plan |
| Unknown | Unknown method |